# Die Cutter Monthly PM

- Procedure

  - Remove tooling. Machine should be empty and everything off the deck. Wear proper PPE, which includes, but is not limited to safety glasses, hearing protection, and safety shoes.

  - Set angle/Position to 69 degrees.

  - Open all windows and doors. Set slip sheet plate height. Pull swords and slip sheet inserter all the way into the blanker.

  - Lockout ALL energy sources.

  - Blowout entire machine (deck up).

    - Feeder.

    - Clean suckers.

  - Clean above Power Register.

    - Clean both sidewalls & everything above Power Register.

  - Clean upper Strip Station.

    - Clean and wipe up everything.

    - Clean behind Platen seals.

  - Clean upper Blanker.

    - Clean and wipe up everything.

    - Move brush towards Blanker.

  - Clean upper Delivery.

    - Use Fireball to wipe up and clean all grease and dust, all around and above.

  - Clean lower Power Register.

    - Clean and wipe everything that you can reach (Fireball and rags).

    - Empty oil collector.

  - Clean Lower Strip Station.

    - Clean and wipe everything you can reach.

  - Clean lower Blanker.

    - Power machine back on (FOLLOW ALL SAFETY PROCEDURES).

    - Push swords and slip sheet inserter back towards home position

    - LOCK OUT

    - Clean and wipe everything you can reach.

    - Wipe photo eyes (Sparkle glass cleaner).

  - Clean Delivery and Rollers.

    - Only remove 3 bolts per board.

    - Keep boards in line when you set them aside

    - Clean out bottom of machine, including pits.

  - Clean Platen Bed.

  - Clean Window and (inside and out).

  - Grease Zerks.

  - Power up machine (FOLLOW SAFETY PROCEDURES).

  - Clean Bernoulli.